| General Specifications |  |
| --- | --- |
| Owner / Operator |  |
| Name |  |
| Type | DPI / AHTSV |
| Flag | VANUATU |
| Year of Built | 2011 |
| Yard / builder | Evergrow Shipbuilding LTD, China |
| Call Sign |  |
| Official No. |  |
| Certificates | IMO, SOLAS, MARPOL, II, STCW |
| Classification | ABS (DP Class I + FIFI I) |
| Measurements / Tonnage |  |
| Length over all | 55M |
| Length P.P. | 50 M |
| Breadth Moulded | 13.8M |
| Draft (maximum) | 4.2M |
| Gross tonnage | 1123 T |
| Net weight | 337 T |
| Deadweight | 1114.79 T |
| Deck |  |
| Free Deck area m2 | Wooden, 350 m2 (25.36 m x 13.8) |
| Deck cargo capacity | 500 TONS |
| Deck strength | 5 TONS / M2 |
| Deck Equipment |  |
| Anchor chain length | 2 X 220 m |
| Anchors type/no./weight | High-hold-type x 2 x 1200 kg |
| Capstans line pull | 2 X 5 tons @ 15m/min |
| Tugger winches: no. x xx T mini | 2 X 10 TONS |
| Anchor handling towing winch and tons capacity. | 1 X Double Drum 120 tons line pull - 1000m x 54mm wire |
| Brake holding on first layer: static holding force | 200 TONS |
| Main Towing winch: Pull force first layer…tons & last layer….tons | 1st layer: 120 tones @ 0-5 m/min |
|  | 2nd layer: 60 tonnes @ 0-10 m/min |
|  | 3rd layer: 15 tonnes @ 0-30 m/min |
| Storage Reels : No. and capacity | Upper drum: 54mm X 1000 m (Towing) |
|  | Lower drum: 54mm X 1000 m (anchor handling) |
| Tow wires: number x length x diameter | 1 X 1000 M X 54 mm |
| Stern roller and winch recess : | 5M X 1.6M (SWL 200 TONS) |
| Retractable shark jaws and towing pin | 1 set X Retractable hydraulic shark Jaws (SWL 200 TONS) |
|  | 1 set vertical hydraulic towing pins |

| Deck Crane xxT/xxm | 1 x electro-hydraulic single boom, 2 tons X 6 m radius |
| --- | --- |
| Power sockets / plugs connected With additional generator | 4 X 415 V, 40 Amp (weather and water proof), 4 X 220 V, 40 Amp (weather and water proof) |
| Passenger Transfer | 2 x landing platforms each side |
| Engines |  |
| Number and type | 2 X CAT 3516B |
| Total BHP | 5150 BHP |
| Bollard Pull | 68.5 Tons - Sustained 69.2 Tons - Max |
| Propellers (type) / Mark | CPP |
| CPP fail safe | NEUTRAL POSITION |
| CPP pump | GEARED ON MAIN ENGINE |
| Rudders (type) | 2 independent |
| Bow thrusters (no. x ….BHP, variable pitch, electrical driven) | 8 TONS THRUST X CPP |
| Stern thrusters (no. …BHP, variable pitch, electrical driven) | 8 TONS THRUST X CPP |
| Service speed knots | 12 KNOTS |
| Shaft generator no x … KVA | 2 X 285 Kw, 415/50/3 |
| Diesel Generator (no. x …KVA | 2 x CAT C18, 285 Kw/415/50/3 |
| Emergency generator (no/el. Power | 1 X 75 Kw |
| Power sockets / plugs | 8 X 415V / 40 amp |
| Tank Capacities |  |
| Fuel m3 | 425 m3 |
| Potable water | 500.4 m3 |
| Drill water | 178 m3 |
| Liquid mud | 138 m3 |
| Lub Oil | 4.5 m3 |
| Ballast water | 178 m3 |
| Discharge lines (no./dia./length) | (3 X 75 mm X 3)" - (1 X 130 mm) - (1 X 100 mm) |
| Couplings (type/diameter) | 4" Camlock |
| Dry Bulk capacities |  |
| Tank number | 4 Vertical tanks |
| Total Tank capacity | 113 m3 |
| Drier | Yes |
| Bulk air dehumidifier | 2 x moisture removal dryers |
| Couplings (type/diameter) | 4" Kamlock |

| Discharge Rates m3/h bar |  |
| --- | --- |
| Fuel (rate/head meter) | 100 m3 / hr @ 75 m head |
| Potable water (rate/head meter) | 100 m3 / hr @ 75 m head |
| Drill water (rate/head meter) | 100 m3 / hr @ 75 m head |
| Mud (rate/head meter) | 2 x 48 m3/hr @t 55 M head |
| Bulk (rate/head meter) | 25 m3 / hr @ 55 m head |
| Miscellaneous Equipment |  |
| Fuel oil meter installed | Yes |
| Freezer (type/capacity) | 1 X 250 liters |
| Cooler (type/capacity) | 1 X 290 liters |
| Reefer container sockets | 4 x 380 - 440 v / 50 HZ |
| Bridge Wheel House Equipment |  |
| Control positions | one forward and one aft |
| Joystick (no./type) | bridge mate |
| Compass | 1 x SAURA KEIKI SEISAKUSHM Model : MR1 50 A |
| Gyro compass | 1 x ANSCHUTZ – Model : DIGITAL STD22 |
| Auto pilot | 1 x COMNAV – Model : COMNAV2001 |
| Radar | 2 X ARPA |
| Echo sounder | 1 x JRC – Model JFE380 |
| Satellite navigator | Yes |
| Accommodation |  |
| Total no. of cabins | 30 persons |
| Crew cabins | 13 persons |
| Passenger cabins (no./beds) | 17 persons |
| Air conditioning | Yes 2 compressors covering 200 %. |
| Navigation & Communication |  |
| Electronic chart system | Available |
| Magnetic compass | 1 x SAURA KEIKI SEISAKUSHM Model : MR1 50 A |
| Gyro compass | 1 x ANSCHUTZ – Model : DIGITAL STD22 |
| DGPS | C- NAV, Dual |
| SSB | 1 x JRC – Model : JPS196GM |
| Navtex and wind sensor | 1 x JRC |
| Radio telex | Available |
| Telefax | Available |
| AIS | 1 x JRC – Model : JHS182 |
| Mobile phone | Available |
| Marine VHF (brand/type) | 2 x JRC – Model : JHS770S |

| Portable VHF (no./type) | (3) x VHF available onboard (standard horizon) model (HX370s) with 40 programmable channels. |
| --- | --- |
| UHF | Available |
| Echo sounder | 1 X JRC – Model : JFE380 |
| Doppler Log | 1 x JRC – Model : JLN205 |
| Mobile emergency VHF | Available |
| Emergency VHF aircraft beacon | Available |
| Emergency auto alarm receiver | Available |

| GPS | 1 x JRC – Model : NWZ4551 |
| --- | --- |
| GMDSS | 2 x JRC – Model : NQE-3154 |
| Inmarsat C (telex, fax, email) | yes |
| Watch receiver/chronometer | yes |
| Radar transponder | (2) 1 x JRC – Model : JMA53126 1 x JRC – Model : JMA2343 |
| Free Float EPIRB/COSPAS | yes |
| Rescue, Safety Equipment |  |
| Rescue zone | Vessel has one dedicated rescue zone on each side. Nets are available. |
| Life jackets (no) | (37) As per SOLAS requirements |
| Life buoys (no) | (8) As per SOLAS requirements |
| Life rafts (no./capacity) | (6) each accommodating 15 persons As per SOLAS requirements |
| Rescue boats (no./type/capacity) | (1) As per SOLAS requirements |
| H2S escape masks | Available |
| Hospital | 2 bunks |
| Fast Rescue Craft for 15 people equipped with jet propulsion | Yes as per SOLAS rescue boats requirements and UKOOA guidelines for standby duties |
| Stretchers | Available |
| Fire Fighting Equipment |  |
| Fire pump (no./capacity) | 2400 m3/hr |
| Fire monitors (no./capacity) | 2 X 1200 m3/hr |
| Foam tank (capacity) | 5 m3 |
| FIFI 1 required (mini 2400m3/h, water throw 120 meter length and vertical height at 45 meter) | Yes |
| Joystick control for fire monitors in wheel house | Yes |

| Anti-Pollution Equipment |  |
| --- | --- |
| Spray booms (no./length) | 2 |
| Dispersant tank (capacity) | Dispersant tank capacity : 12 m3 ~ (13.82 MT) The dispersant system with a built-in tank with (1) bow throw spray nozzles each (P&S). |
| Speed / Consumption |  |
| Max speed / consumption | 13 knots |
| Service speed / consumption in m3 per day | 12 knots |
| Composition of crew on board |  |
| Manning | Complied |
| Rotation schedule for each category of above personnel | Complied |